VAYA CONNECT

Product & Technical
Overview

Caregiver control, safety workflows, and ESP32 controller architecture for VAYA One.

| Caregiver-first Fast, clear control for a companion operating a pediatric mobility device. | Safety-led Communication loss, stale commands, charging, and faults move the chair to a safe state. | Production path Structured BLE protocol, state machine, trusted devices, diagnostics, and scalable configuration. |
| --- | --- | --- |

# 1. Product Overview

VAYA Connect is the primary mobile companion interface for VAYA One, a smart pediatric wheelchair, specially designed for dystonia/cerebral palsy patients. The caregiver uses it to connect, verify safety state, control movement, manage trusted phones, respond to emergencies, and inspect the wheelchair's real telemetry.

## Caregiver-facing capability map

| Area | What the caregiver sees | How it works |
| --- | --- | --- |
| Secure connection | Guided setup, VAYA One discovery, passkey entry, clear recovery messages. | BLE pairing uses a six-digit passkey and trusted-device policy. The app waits for protocol negotiation before enabling controls. |
| Driving | Indoor / Outdoor modes, virtual joystick, sensitivity control, battery indicator, connection and motion state. | The joystick sends bounded, sequenced movement commands while held. Release, lifecycle interruption, or safety loss sends/causes a stop. |
| Locking | Visible locked/unlocked state and a clear action button. | Unlock and passkey change require device authentication. The firmware permits lock changes only when motor output is at rest. |
| Emergency | Emergency Stop; separate SOS assistance; emergency contacts and message setup. | Emergency Stop latches a motion stop. SOS opens native call/SMS assistance with an optional current-location link. |
| Monitoring | Connection, movement, battery, speed, charging, faults, diagnostics, pre-drive checklist. | The app renders controller telemetry only when fresh. Unavailable data is shown as unavailable rather than guessed. |

## Design principles

Safety before appearance: every control communicates the present state and the consequence of an action.

One primary purpose per screen: Control drives, Status verifies, and Settings manages configuration and diagnostics.

Acknowledged operations: the app does not assume a sent BLE command was accepted; critical commands await controller acknowledgement.

Responsive mobile operation: Control has been tested for small-phone layouts and enlarged text without requiring scrolling.

# 2. Main Caregiver Workflows

## 2.1 Connect a wheelchair

Open VAYA Connect. Permission guidance is shown only when Bluetooth or location access is actually required.

Scan nearby devices. The app de-duplicates scan results and presents compatible VAYA One controllers.

Select the wheelchair and enter its six-digit passkey. iOS/Android system pairing completes secure BLE bonding.

The app sends HELLO and acquires a zero-output control lease. The chair remains locked after connection.

The Status and Control screens become active only after live telemetry and protocol acknowledgement are available.

## 2.2 Drive safely

Use Status to confirm connection, lock state, stopped state, no active fault/charging, and battery telemetry.

Unlock the chair only when stopped. Face ID, fingerprint, or device passcode confirms this sensitive action.

Choose Indoor for low-speed precision or Outdoor for the configured balanced-power limit.

Hold and move the virtual joystick. The app sends live movement values at a fixed cadence while it is held.

Release the joystick to request stop. The controller also stops automatically on stale commands, lease expiry, disconnect, or fault.

## 2.3 Emergency assistance

| Trigger | Immediate behavior | Caregiver outcome |
| --- | --- | --- |
| Emergency Stop | High-priority ESTOP command is sent; controller latches a safe stopped state. | Movement stays blocked until the required physical reset conditions are met. |
| Physical SOS hold | ESP32 emits an SOS telemetry event; the app displays a non-dismissable alert until acknowledged. | With no contacts, the app opens the 112 call flow. With contacts, it offers the configured call target and a prepared message. |
| Digital SOS | The app opens the same assistance sheet without changing the motor state. | Caregiver can call the selected target or open a prepared SMS to all contacts. |

Platform behavior The app opens the native phone dialer and SMS composer. The caregiver gives the operating system's final call/send confirmation; calls and texts are not silently sent by the app.

# 3. System Architecture

The system deliberately separates mobile presentation, mobile state/orchestration, BLE transport, command validation, safety state, motor output, and optional sensors. Each layer has a narrow responsibility.

## Logical architecture

| VAYA Connect UI Control, Status, Settings, emergency assistance, accessible feedback. | AppState + Services Permissions, BLE acknowledgement waiting, telemetry freshness, local emergency plan, device authentication. | BLE V2 transport Encrypted GATT connection, framed commands, CRC, sequence numbers, acknowledgements, telemetry notifications. | ESP32 controller State machine, watchdogs, motor controller, trust policy, sensors, standby, logs. |
| --- | --- | --- | --- |

## App modules

| Module | Responsibility | Key behavior |
| --- | --- | --- |
| Screens and widgets | Caregiver interaction | Control has no scrolling dependency; Status is a readable, scrollable monitoring surface; Settings separates advanced actions. |
| AppState | UI state and orchestration | Tracks connection, acknowledgements, telemetry freshness, safety conditions, SOS acknowledgement, and locked/unlocked state. |
| Bluetooth service | BLE lifecycle | Discovers, connects, streams telemetry, coordinates protocol frames, and restores active connections where possible. |
| Emergency assistance service | Local emergency configuration | Stores contacts, chosen call target, and message locally; requests live phone location and opens native call/SMS flows. |
| Local authentication | Sensitive-action confirmation | Requires device passcode/biometric confirmation before unlock and passkey rotation. |

## Firmware modules

BleTransport: encrypted BLE server, bond management, pairing policy, advertisements, command queue, and telemetry notifications.

DeviceController: authoritative lock, connection, lease, fault, and emergency-stop state machine.

MotorController: PWM ramping, direction dead time, dead zone, motor-output snapshot, safe stop, and bounded output limits.

Sensor services: battery availability/calibration, optional charger detection, Pair/Wake button, optional SOS button, and optional alert output.

Logger: change-based serial diagnostics with safety work prioritized before logging and telemetry.

# 4. Controller State Machine

The ESP32 always exposes one explicit device state. Safety events transition toward a locked, stopped, fault, or emergency-stop state; no event should leave a stale motor request active.

## State definitions

| State | Meaning | Allowed path out |
| --- | --- | --- |
| BOOTING | Startup initialization. Motor outputs are immediately set safe. | Initialization success transitions to DISCONNECTED; configuration failure raises FAULT. |
| DISCONNECTED | No authenticated caregiver BLE session. Chair is locked and motors are stopped. | Authenticated connection enters CONNECTED_LOCKED. |
| CONNECTED_LOCKED | Secure session exists, but driving is disabled. | HELLO + ACQUIRE + safe unlock enters CONNECTED_IDLE. |
| CONNECTED_IDLE | Secure lease valid, chair unlocked, output at rest. | Valid MOVE enters MOVING; LOCK enters CONNECTED_LOCKED. |
| MOVING | A bounded drive request is active. | Zero/release/STOP moves toward STOPPING or IDLE; watchdog/lease/disconnect locks and stops. |
| STOPPING | Motor ramp-down or direction dead-time is in progress. | At rest transitions to CONNECTED_IDLE or CONNECTED_LOCKED. |
| FAULT | A non-emergency safety fault is active; motors stopped and chair locked. | A valid recovery sequence may clear eligible faults through control acquisition. |
| EMERGENCY_STOP | Emergency stop is latched; motors stopped and chair locked. | Requires physical E-stop release, motor rest, and reset conditions. |

## Core state transitions

| Event | Transition | Safety action |
| --- | --- | --- |
| BLE connects | DISCONNECTED → CONNECTED_LOCKED | Motor emergency stop; protocol and lease reset; chair remains locked. |
| Acquire control | CONNECTED_LOCKED → CONNECTED_LOCKED | Creates a short control lease but preserves lock and zero motor output. |
| Safe unlock | CONNECTED_LOCKED → CONNECTED_IDLE | Requires valid lease and fully rested motor snapshot. |
| Valid drive request | CONNECTED_IDLE → MOVING | Applies bounded left/right motor request under current PWM limit. |
| Release / stop | MOVING → STOPPING → IDLE | Cancels motion request and ramps motor output safely to zero. |
| Stale movement command | MOVING → CONNECTED_LOCKED | Motion watchdog emergency-stops motors and records COMMAND_TIMEOUT. |
| Lease expiry / disconnect | Any active state → CONNECTED_LOCKED or DISCONNECTED | Stops motors, clears lease, and locks movement. |
| Physical E-stop / ESTOP | Any state → EMERGENCY_STOP | Latches emergency state and requires physical-safe reset conditions. |

# 5. BLE Protocol, Telemetry, and Safety Interlocks

## 5.1 Command contract

Commands are text frames with V2 versioning, a monotonically increasing sequence number, and CRC-16 integrity validation. The controller returns an acknowledgement containing the sequence, result, and actual controller state.

Command: V2|C|<sequence>|<command>|<arguments>|<CRC16>
Ack:     V2|A|<sequence>|<result>|<controller_state>|<CRC16>
Telemetry: V2|T|<sequence>|<uptime>|...|<firmware_version>|<CRC16>

| Command | Purpose | Important acceptance rule |
| --- | --- | --- |
| HELLO / ACQUIRE | Negotiate protocol and obtain a short control lease. | Acquire begins with the chair locked and motor output at zero. |
| MOVE | Set normalized left/right drive intent. | Requires valid lease, unlocked chair, no fault, and no charging state. |
| STOP / ESTOP | Request normal stop or latch emergency stop. | STOP is always processed; ESTOP overrides ordinary control and latches safe state. |
| LOCK | Lock or unlock movement. | Both directions require motors at rest and no pending motion target. Unlock is rejected while charging. |
| MODE / LIMIT | Apply indoor/outdoor or bounded PWM limit. | Requires a valid control lease. |
| PASSKEY / FORGET / ALLOW_NEW | Security and trusted-device management. | Requires a secure lease and stationary motor state. Passkey rotation invalidates prior bonds. |

## 5.2 Telemetry contract

Controller state, lock state, emergency-stop state, fault code, left/right PWM output, and motion context.

Battery sensor state, raw ADC value, calibrated millivolts, and percentage only when the sensor is valid and calibrated.

Trusted-device count, new-device policy, physical pairing-window state, firmware version, charger availability/state, and SOS active state.

Telemetry is sent every 250 ms while connected. The app marks battery and movement data delayed after three seconds and unavailable when absent.

## 5.3 Safety timers and interlocks

| Control | Current setting | Behavior |
| --- | --- | --- |
| Motion watchdog | 500 ms | If a non-zero motion command becomes stale, motors are emergency-stopped, the chair locks, and COMMAND_TIMEOUT is recorded. |
| Control lease | 2 seconds | If control is not refreshed, motors stop and the chair locks even if the BLE link remains connected. |
| Motor update | 10 ms | PWM output updates with ramping and direction-change dead time. |
| Direction dead time | 60 ms | Prevents direct reversal from one motor direction to the opposite direction. |
| Pairing hold/window | 5 seconds / 120 seconds | A configured physical Pair/Wake button opens a temporary pairing opportunity only when stopped and locked. |
| SOS hold/active | 2 seconds / 120 seconds | A configured physical SOS hold raises an event and can drive an external alert indicator. |
| Auto standby | 10 minutes | When configured and eligible, BLE advertising stops only while locked, stationary, disconnected, and fault-free. |

# 6. Security, Emergency, and Operating Flows

## 6.1 Trusted-device model

A BLE bond represents a trusted caregiver phone. The controller counts trusted bonds independently of what a particular phone remembers.

A chair with no trusted bond remains recoverable for initial setup. Once trusted bonds exist, a new device is blocked unless the policy allows it and, in production, a physical pairing window is open.

Changing the passkey updates secure storage, invalidates all existing bonds, locks the chair, and disconnects the current session.

Removing a wheelchair from the app removes the current caregiver bond when the controller confirms it. iPhone system Bluetooth records may still need manual removal after a passkey change or removal.

## 6.2 Emergency assistance flow

Caregiver configures contacts, a selected call target (112 by default or one contact), and a reusable message template containing {location}.

Physical SOS: the ESP32 detects a configured button hold, marks SOS active, wakes BLE, and includes SOS in telemetry.

App receives a rising SOS event, marks it as requiring acknowledgement, and opens the emergency-assistance surface.

App requests the phone's current location with a short timeout. It uses a last-known location when available; otherwise it states that location is unavailable.

The app opens native phone/SMS surfaces. The final call/send confirmation remains with the caregiver and operating system.

## 6.3 Charging flow

When the optional charger-detect input reports charging, the controller immediately stops motors, clears the control lease, locks the chair, and sends charging telemetry. Driving and unlocking are rejected until charging clears; the chair remains locked after charging ends until a caregiver intentionally unlocks it.

## 6.4 Standby and wake flow

Auto standby is optional and requires the physical Pair/Wake button. After the configured idle interval, only a disconnected, locked, stationary, fault-free chair can stop advertising. A short Pair/Wake press restores advertising while keeping the chair locked. An SOS event also wakes BLE so the alert can reach the phone.

Operating limitation A BLE SOS event reaches the app only while the phone is connected and permitted to receive the connection. A future backend emergency relay is needed for alerts when the caregiver phone is unavailable, out of range, or cannot maintain background BLE operation.

# 7. Configuration, Test, and Release Guidance

## 7.1 Build-time feature flags

Core motor pins and safety limits are fixed in firmware configuration. Optional prototype hardware is enabled through build-time environment values so an app-only or bench build does not claim hardware exists.

VAYA_FEATURE_PAIRING_BUTTON=true
VAYA_PAIR_WAKE_BUTTON_PIN=<GPIO>
VAYA_FEATURE_SOS_BUTTON=true
VAYA_SOS_BUTTON_PIN=<GPIO>
VAYA_FEATURE_CHARGER_DETECTION=true
VAYA_CHARGER_DETECT_PIN=<GPIO>

| Feature | Bench test | Production requirement |
| --- | --- | --- |
| Pair / Wake | Use a momentary switch to ground on the configured active-low input. | Enclosure-accessible button, debounce verification, and controlled pairing procedure. |
| SOS | Use a momentary switch; verify a two-second hold triggers only one telemetry event. | Large accessible hold button, tactile marking, local feedback, and caregiver response procedure. |
| Charging detect | Use an isolated logic-level test signal; never place charger voltage on an ESP32 GPIO. | Isolated or level-shifted BMS/charger status line, validation under charger faults, and drive inhibition testing. |
| Motor output | Test with propulsion hardware isolated; inspect logged left/right PWM values. | Validated motor driver, current protection, physical E-stop hardware, and controlled mobility-device test protocol. |

## 7.2 Recommended next engineering work

| Priority | Work item | Reason |
| --- | --- | --- |
| 1 | Independent motor-enable emergency-stop circuit | Ensures a hardware stop path does not depend on BLE, Flutter, or ESP32 firmware. |
| 2 | Physical joystick filtering and validation | Addresses involuntary input, twitch rejection, dead zone, acceleration limits, and neutral-before-resume behavior. |
| 3 | Motor feedback / thermal / current sensing | Enables stall, driver fault, over-temperature, and over-current protection. |
| 4 | Real BMS integration | Provides trustworthy battery voltage, charge state, current, temperature, and fault information. |
| 5 | Backend emergency relay | Supports alerts when the caregiver phone is unavailable. |